| PINNACLE HEALTH CLINIC NOC - Network Incident Ticket | PHC-NET-005 06/11/2026 |
| --- | --- |

| Ticket # | PHC-NET-005 | Priority | High |
| --- | --- | --- | --- |
| Category | Network Connectivity - Printer Offline | Date | 06/11/2026 09:45 |
| Reported By | Dan Foster - Billing Dept | Device | PHC-PRINT01 (192.168.40.22) |
| Assigned To | Will Turner - IT Administrator |  |  |
| Status | CLOSED - RESOLVED |  |  |

Incident Description

Dan Foster (Billing Department) called the NOC at 09:45 reporting that the shared network printer PHC-PRINT01 was showing offline and no department in the clinic could print. Multiple staff members across Clinical, Admin, and Billing had attempted to print over the previous 20 minutes with no success. The incident was classified as High priority due to clinic-wide impact on printing operations.

NOTE: PHC-PRINT01 is a shared network printer serving all departments - Clinical, Admin, Billing, and IT. Printing is required for patient intake forms, billing documents, and clinical records. Extended printer outage impacts all departments simultaneously.

Diagnostic Process

Diagnostic methodology: OSI model troubleshooting working from Layer 1 upward. Each test was designed to isolate the layer where the fault exists.

1.  Confirmed with Dan Foster that all departments were affected - not a single workstation issue. Clinic-wide printer outage points to a network-side fault rather than a driver or workstation problem.

2.  Pinged PHC-PRINT01 (192.168.40.22) from Admin-PC01 - 100% loss, 4/4 Request timed out. Printer is completely unreachable on the network.

3.  Key observation: printer has a static IP (192.168.40.22) on VLAN 40. DHCP failure is not a factor. The device is either powered off or the switch port is down.

4.  Accessed SW-CORE CLI - ran show vlan brief. Confirmed Gig1/0/7 is still assigned to VLAN 40 (IT-Servers). VLAN assignment is correct - not a VLAN issue.

5.  Ran show interface gigabitethernet 1/0/7 - output showed: GigabitEthernet1/0/7 is administratively down, line protocol is down (disabled). Port was manually shut down.

6.  ROOT CAUSE IDENTIFIED: SW-CORE port Gig1/0/7 connecting PHC-PRINT01 was administratively shut down. Consistent with a technician running shutdown on the wrong interface during a maintenance window.

OSI Layer Analysis

| Layer | Name | Test Result | Conclusion |
| --- | --- | --- | --- |
| 1 | Physical | show interface Gig1/0/7 - administratively down | FAULT HERE - port shut down on SW-CORE |
| 2 | Data Link | show vlan brief - Gig1/0/7 in VLAN 40 | VLAN assignment correct - not a Layer 2 config issue |
| 3 | Network | ping 192.168.40.22 - 100% loss | Direct result of Layer 1 fault - no physical link |
| 4-7 | Transport-App | Not tested | N/A until Layer 1 resolved |

Root Cause Analysis

| Root cause | SW-CORE port Gig1/0/7 was administratively shut down |
| --- | --- |
| Impact | PHC-PRINT01 lost all network connectivity - all departments unable to print for approximately 20 minutes before ticket was raised |
| Likely cause | Technician ran shutdown on Gig1/0/7 in error during a maintenance window - intended to shut down an unused port but targeted the printer port instead |
| Key insight | A shutdown switch port is invisible from the device side - PHC-PRINT01 showed no error, it simply lost link. The fault is only visible from the switch via show interface. When a network device goes offline with no warning and ping fails, check the access switch port status before investigating the device itself. |

Resolution

1.  Accessed SW-CORE CLI.

2.  Entered: enable > configure terminal > interface gigabitethernet 1/0/7 > no shutdown > end

3.  Console confirmed: %LINK-5-CHANGED: Interface GigabitEthernet1/0/7, changed state to up

4.  Ran show interface gigabitethernet 1/0/7 - confirmed GigabitEthernet1/0/7 is up, line protocol is up (connected).

5.  Pinged PHC-PRINT01 from Admin-PC01 - 3/4 success, TTL 127 (1 drop = ARP resolution, normal).

6.  Confirmed with Dan Foster that printer is back online and printing is restored across all departments.

7.  Documented incident and closed ticket.

Verification

| Verification Check | Result | Notes |
| --- | --- | --- |
| show interface Gig1/0/7 - port status | PASS | GigabitEthernet1/0/7 is up, line protocol is up |
| Ping PHC-PRINT01 192.168.40.22 | PASS | 3/4 packets, TTL 127 (1 drop = ARP, normal) |
| Printer accessible from all VLANs | PASS | Inter-VLAN routing to VLAN 40 confirmed working |
| User confirmed printing restored | PASS | Confirmed with Dan Foster - Billing Dept |

Recommendations

- Add port shutdown verification to the post-maintenance checklist - run show interface on all active device ports before closing any change window.

- Label all switch ports in the patch panel with the connected device name and IP - a physical label on Gig1/0/7 reading PHC-PRINT01 would have prevented this error.

- Create a port inventory document mapping all SW-CORE ports to connected devices - technicians should reference this before running any shutdown command.

- Consider implementing port descriptions in IOS config - switchport description PHC-PRINT01-192.168.40.22 on Gig1/0/7 would make the port purpose visible in show interface output.

Incident Timeline

| Time | Event |
| --- | --- |
| Previous evening | SW-CORE maintenance window - Gig1/0/7 shut down in error |
| 09:25 | Staff begin reporting printer offline across all departments |
| 09:45 | Dan Foster calls NOC - clinic-wide printer outage reported |
| 09:45 | Will Turner begins diagnosis - ping 192.168.40.22 returns 100% loss |
| 09:55 | show vlan brief confirms VLAN 40 assignment correct on Gig1/0/7 |
| 09:59 | show interface Gig1/0/7 - administratively down confirmed - root cause identified |
| 10:04 | no shutdown applied to Gig1/0/7 - interface comes up immediately |
| 10:06 | Verification ping to 192.168.40.22 succeeds - printer back online |
| 10:15 | Dan Foster confirms printing restored across all departments - ticket closed |

Technician Sign-Off

| Analyst | Will Turner - IT Administrator | Time to Resolve | 30 minutes |
| --- | --- | --- | --- |
| Ticket Closed | 06/11/2026 10:15 | User Confirmed | Yes - Dan Foster |

This ticket is part of the Pinnacle Health Clinic IT Portfolio - a simulated healthcare IT environment built for professional development and portfolio documentation.

Pinnacle Health Clinic - NOC Network Incident Documentation - Confidential