| Ticket # | PHC-NET-001 | Priority | High |
| --- | --- | --- | --- |
| Category | Network Connectivity | Date | 06/06/2026 08:47 |
| Reported By | Dr. James Carter | Workstation | PC-CLIN01 |
| Assigned To | NOC Analyst on duty - Pinnacle Health NOC |
| Status | CLOSED - RESOLVED |

INCIDENT DESCRIPTION

Dr. James Carter called the NOC at 08:47 reporting complete loss of network connectivity on PC-CLIN01. He was unable to access the patient records system and had a patient waiting. The incident was classified as High priority due to direct impact on patient care operations.

HIPAA NOTE: PC-CLIN01 is a clinical workstation with access to patient records. Any connectivity failure on this device directly impacts patient care continuity and must be treated as high priority.

DIAGNOSTIC PROCESS

Diagnostic methodology: OSI model troubleshooting working from Layer 1 upward. Each test was designed to isolate the layer where the fault exists.

1.  Ran ipconfig on PC-CLIN01. Output showed IP 192.168.10.10, Subnet 255.255.255.0, Default Gateway 0.0.0.0 - gateway missing.

2.  Pinged self (192.168.10.10) - 4/4 success. Confirmed Layer 1 and 2 operational - NIC working, switch port active, IP stack functional.

3.  Pinged gateway (192.168.10.1) - 4/4 success. Gateway is on the same subnet (192.168.10.0/24) so no gateway is required to reach it directly via ARP.

4.  Pinged SERVER01 (192.168.40.10) - 0/4 failed, Request timed out. SERVER01 is on VLAN 40 - a different subnet. Without a default gateway the PC has no path to route the packet.

5.  Pinged simulated WAN (203.0.113.1) - 0/4 failed. Confirmed no connectivity beyond the local subnet.

6.  ROOT CAUSE IDENTIFIED: Default gateway missing from PC-CLIN01 IP configuration. Local subnet connectivity intact but all inter-subnet routing broken.

OSI LAYER ANALYSIS

| Layer | Name | Test Result | Conclusion |
| --- | --- | --- | --- |
| 1 | Physical | NIC active, green link on switch | OK - no issue |
| 2 | Data Link | Ping 192.168.10.10 success | OK - no issue |
| 3 | Network | Ping 192.168.40.10 failed - 0.0.0.0 gateway | FAULT HERE - missing default gateway |
| 4-7 | Transport-App | Not tested - Layer 3 fault blocks higher layers | N/A until Layer 3 resolved |

ROOT CAUSE ANALYSIS

| Field | Detail |
| --- | --- |
| Root cause | Default gateway missing from PC-CLIN01 IP configuration (0.0.0.0) |
| Impact | No connectivity to any subnet outside 192.168.10.0/24 - patient records system unreachable |
| Likely cause | User or technician manually switched from DHCP to static IP without entering gateway, or DHCP lease failed to renew |
| Key insight | Missing gateway does not break local subnet connectivity - only inter-subnet routing fails. User could still reach devices in VLAN 10 but not patient records server in VLAN 40. |

RESOLUTION

1.  Switched PC-CLIN01 IP configuration from Static back to DHCP.

2.  DHCP lease obtained from SERVER01 (192.168.40.10) via ip helper-address on SW-CORE VLAN 10 SVI.

3.  New IP assigned: 192.168.10.12, Default Gateway: 192.168.10.1, DNS: 192.168.40.10.

4.  Verified with ipconfig - Default Gateway now populated correctly.

5.  Ran verification pings: 192.168.10.1 (gateway), 192.168.40.10 (SERVER01), 203.0.113.1 (WAN) - all successful.

6.  Confirmed with Dr. Carter that patient records system is accessible.

7.  Documented incident and closed ticket.

VERIFICATION

| Verification Check | Result | Notes |
| --- | --- | --- |
| Default gateway populated via DHCP | PASS | 192.168.10.1 confirmed in ipconfig |
| Ping gateway 192.168.10.1 | PASS | 4/4 packets, 0% loss |
| Ping SERVER01 192.168.40.10 | PASS | 4/4 packets, TTL 127 |
| Ping WAN 203.0.113.1 | PASS | 4/4 packets, TTL 254 |
| Patient records system accessible | PASS | Confirmed with Dr. Carter |
| DHCP lease stable | PASS | IP 192.168.10.12 assigned |

RECOMMENDATIONS

- Enforce DHCP on all clinical workstations via Group Policy - prevent users from switching to static IP without IT authorization.

- Set up DHCP lease monitoring on SERVER01 - alert NOC if VLAN 10 lease count drops unexpectedly.

- Add PC-CLIN01 to Wazuh monitoring - network connectivity loss events should generate an alert before the user calls.

- Document in knowledge base: missing default gateway symptoms - local connectivity intact, cross-subnet fails. Common cause is manual static IP change.

INCIDENT TIMELINE

| Time | Event |
| --- | --- |
| 08:47 | Dr. Carter calls NOC reporting no network connectivity on PC-CLIN01 |
| 08:48 | NOC analyst begins remote diagnosis - runs ipconfig, identifies missing gateway |
| 08:49 | Diagnostic ping sequence confirms inter-subnet routing broken, local subnet intact |
| 08:50 | Root cause confirmed - static IP configuration missing default gateway |
| 08:51 | Fix applied - switched to DHCP, gateway restored, connectivity verified |
| 08:52 | Dr. Carter confirms patient records accessible - ticket closed |

TECHNICIAN SIGN-OFF

| Analyst | NOC Analyst on duty | Time to Resolve | 5 minutes |
| --- | --- | --- | --- |
| Ticket Closed | 06/06/2026 08:52 | User Confirmed | Yes - Dr. Carter |

This ticket is part of the Pinnacle Health Clinic IT Portfolio - a simulated healthcare IT environment built for professional development and portfolio documentation.