| PINNACLE HEALTH CLINIC NOC - Network Incident Ticket | PHC-NET-002 06/07/2026 |
| --- | --- |

| Ticket # | PHC-NET-002 | Priority | High |
| --- | --- | --- | --- |
| Category | Network Connectivity | Date | 06/07/2026 08:47 |
| Reported By | Tom Ellis - Front Desk, Admin Dept | Workstation | Admin-PC01 |
| Assigned To | Will Turner - IT Administrator |  |  |
| Status | CLOSED - RESOLVED |  |  |

Incident Description

Tom Ellis (Front Desk, Admin Department) called the NOC at 08:47 reporting complete loss of network connectivity. He was unable to access the scheduling system or shared drive after booting his PC. The incident was classified as High priority due to direct impact on front desk operations.

HIPAA NOTE: The Admin VLAN (VLAN 20) handles scheduling and patient appointment systems. Connectivity failure on any Admin workstation can impact patient intake and scheduling continuity and must be treated as high priority.

Diagnostic Process

Diagnostic methodology: OSI model troubleshooting working from Layer 1 upward. Each test was designed to isolate the layer where the fault exists.

1.  Ran ipconfig on Admin-PC01. The PC still had its previous lease showing (192.168.20.10) but gateway was showing blank - lease was about to expire.

2.  Ran ipconfig /release - IP address dropped to 0.0.0.0 as expected.

3.  Ran ipconfig /renew - returned DHCP request failed. The DHCP broadcast from VLAN 20 was not reaching SERVER01. Root cause suspected immediately.

4.  Pinged gateway (192.168.20.1) - 100% loss. PC had no valid IP address - could not reach anything.

5.  Pinged SERVER01 (192.168.40.10) - 100% loss. No inter-VLAN connectivity confirmed.

6.  Pinged simulated WAN (203.0.113.1) - 100% loss. Confirmed complete network isolation.

7.  Accessed SW-CORE CLI - ran show running-config. Compared all SVI configurations. VLANs 10, 30, 40, and 99 all had ip helper-address 192.168.40.10 configured. VLAN 20 SVI was missing the entry entirely.

8.  ROOT CAUSE IDENTIFIED: ip helper-address missing from interface Vlan20 on SW-CORE. DHCP broadcasts from the Admin VLAN cannot reach SERVER01. Consistent with maintenance performed on SW-CORE the previous evening.

OSI Layer Analysis

| Layer | Name | Test Result | Conclusion |
| --- | --- | --- | --- |
| 1 | Physical | NIC active, link connected | OK - no issue |
| 2 | Data Link | Switch port in VLAN 20 confirmed | OK - no issue |
| 3 | Network | ipconfig /renew - DHCP request failed | FAULT HERE - ip helper-address missing from Vlan20 SVI on SW-CORE |
| 4-7 | Transport-App | Not tested | N/A until Layer 3 resolved |

Root Cause Analysis

| Root cause | ip helper-address missing from interface Vlan20 on SW-CORE |
| --- | --- |
| Impact | DHCP broadcasts from VLAN 20 (Admin) could not reach SERVER01 - entire Admin VLAN lost DHCP at next boot cycle |
| Likely cause | SW-CORE maintenance window the previous evening - VLAN 20 SVI was reconfigured and ip helper-address was not restored after maintenance completed |
| Key insight | DHCP operates by broadcast. Broadcasts do not cross VLAN boundaries. The ip helper-address on the SVI converts the DHCP broadcast to a unicast forwarded to SERVER01. Without it, no device on VLAN 20 can obtain a lease. The switch itself shows no error - the failure is silent at the switch level. |

Resolution

1.  Accessed SW-CORE CLI.

2.  Entered: enable > configure terminal > interface vlan 20 > ip helper-address 192.168.40.10 > end

3.  Returned to Admin-PC01 - ran ipconfig /renew - DHCP lease obtained from SERVER01 immediately.

4.  New IP assigned: 192.168.20.10, Default Gateway: 192.168.20.1, DNS: 192.168.40.10.

5.  Ran verification pings: 192.168.20.1 (gateway), 192.168.40.10 (SERVER01), 203.0.113.1 (WAN) - all successful.

6.  Confirmed with Tom Ellis that scheduling system and shared drive are accessible.

7.  Documented incident and closed ticket.

Verification

| Verification Check | Result | Notes |
| --- | --- | --- |
| DHCP lease obtained via ip helper-address relay | PASS | 192.168.20.10 assigned from SERVER01 |
| Ping gateway 192.168.20.1 | PASS | 4/4 packets, 0% loss |
| Ping SERVER01 192.168.40.10 | PASS | 4/4 packets, TTL 127 |
| Ping WAN 203.0.113.1 | PASS | 3/4 packets, TTL 254 (1 drop = ARP, normal) |
| Scheduling system accessible | PASS | Confirmed with Tom Ellis |
| DHCP lease stable | PASS | IP 192.168.20.10 assigned and stable |

Recommendations

- Add ip helper-address verification to the post-maintenance checklist for all SW-CORE SVI changes - confirm all five VLANs (10, 20, 30, 40, 99) have the relay before closing any change window.

- Set up DHCP lease monitoring on SERVER01 - alert NOC if VLAN 20 lease count drops unexpectedly.

- Document in knowledge base: DHCP request failed after a maintenance window is a strong indicator of missing ip helper-address. Check show running-config on the affected VLAN SVI first.

- Consider adding Admin-PC01 to Wazuh monitoring - network connectivity loss events should generate an alert proactively.

Incident Timeline

| Time | Event |
| --- | --- |
| Previous evening | SW-CORE maintenance window - VLAN 20 SVI reconfigured, ip helper-address not restored |
| 08:47 | Tom Ellis calls NOC - no network connectivity after boot |
| 08:47 | Will Turner begins diagnosis - ipconfig /renew returns DHCP request failed |
| 08:48 | All three diagnostic pings fail - gateway, SERVER01, and WAN all unreachable |
| 08:49 | show running-config on SW-CORE - ip helper-address confirmed missing from Vlan20 SVI only |
| 08:50 | ip helper-address 192.168.40.10 restored to interface Vlan20 |
| 08:51 | ipconfig /renew on Admin-PC01 - valid DHCP lease obtained from SERVER01 |
| 08:52 | Tom Ellis confirms scheduling system accessible - ticket closed |

Technician Sign-Off

| Analyst | Will Turner - IT Administrator | Time to Resolve | 5 minutes |
| --- | --- | --- | --- |
| Ticket Closed | 06/07/2026 08:52 | User Confirmed | Yes - Tom Ellis |

This ticket is part of the Pinnacle Health Clinic IT Portfolio - a simulated healthcare IT environment built for professional development and portfolio documentation.

Pinnacle Health Clinic - NOC Network Incident Documentation - Confidential