| PINNACLE HEALTH CLINIC NOC - Network Hardening Implementation | PHC-NET-006 06/20/2026 |
| --- | --- |

| Ticket # | PHC-NET-006 | Priority | HIGH |
| --- | --- | --- | --- |
| Category | Network Segmentation - VLAN Hardening | Date | 06/20/2026 |
| Requested By | Clinic Management - Post-Audit Recommendation | System | SW-CORE, SW-SECURITY (Packet Tracer) |
| Assigned To | Willie Conaway (wconaway) - Cybersecurity Consultant |  |  |
| Status | CLOSED - RESOLVED |  |  |

Project Description

As part of the hardening plan opened following PHC-CYBER-002/003, the two remaining medium-priority hardening items were implemented to complete network-level segmentation: physical security camera infrastructure on an isolated VLAN, and guest wireless network isolation. Both items had been deferred from the original VirtualBox hardening session (PHC-CYBER-004) since they require Packet Tracer network changes rather than Windows Server configuration. During the same session, RFID-based physical access control was also added for the server room door and is documented here as a related access control enhancement.

HIPAA NOTE: Network segmentation of non-clinical IoT devices (security cameras) and untrusted networks (guest wireless) from systems handling PHI is a required safeguard under 45 CFR 164.312(a)(1) (Access Control) and 164.312(e)(1) (Transmission Security). A compromised camera or guest device must not be able to reach clinical, billing, or administrative systems.

Item 9 - Security Camera VLAN

| Field | Detail |
| --- | --- |
| New VLAN | VLAN 60 - Security_Cameras - 192.168.60.0/24 |
| New Switch | SW-SECURITY (Cisco 2960) - new dedicated access switch |
| Devices Added | 6 cameras - CAM-LOBBY01, CAM-LOBBY02, CAM-HALL01, CAM-HALL02, CAM-BACKDOOR01, CAM-SERVERRM01 |
| Camera Ports | SW-SECURITY Fa0/2 through Fa0/7 - access mode, VLAN 60 |
| Uplink | SW-SECURITY Fa0/1 to SW-CORE Gig1/0/8 - trunk, VLAN 60 allowed |
| DHCP | New scope added on Server01 for 192.168.60.0/24 - gateway 192.168.60.1 |
| Isolation Control | ACL CAMERA-ISOLATION applied inbound on Vlan60 SVI - permits Server01 and DHCP only, denies Clinical/Admin/Billing/Guest |

Item 10 - Guest Network Isolation

| Field | Detail |
| --- | --- |
| Target VLAN | VLAN 99 - Guest - 192.168.99.0/24 |
| Policy | Guest devices restricted to WAN access only - no access to internal department VLANs or the camera VLAN |
| Isolation Control | ACL GUEST-ISOLATION applied inbound on Vlan99 SVI - denies Clinical, Admin, Billing, IT, and Camera subnets, permits DHCP renewal and all other traffic (WAN) |
| Rationale | Guest wireless is the least trusted network segment in the clinic. A compromised or malicious guest device must not be able to reach PHI systems, billing records, or camera infrastructure. |

Implementation Steps - Item 9 (Security Cameras)

1.  Added SW-SECURITY (Cisco 2960) switch to the Packet Tracer topology, connected to SW-CORE Gig1/0/8.

2.  Created VLAN 60 on SW-CORE - name Security_Cameras - configured SVI with IP 192.168.60.1/24 and ip helper-address 192.168.40.10.

3.  Configured SW-CORE Gig1/0/8 as a trunk port allowing VLAN 60, descriptioned SW-SECURITY-TRUNK.

4.  Configured SW-SECURITY Fa0/1 as a trunk port to SW-CORE, Fa0/2 through Fa0/7 as access ports in VLAN 60.

5.  Added 6 camera devices to SW-SECURITY representing lobby, hallway, back door, and server room coverage.

6.  Diagnosed initial DHCP failure - root cause was a missing DHCP scope for 192.168.60.0/24 on Server01 (all switch-side configuration was already correct). Added the scope with gateway 192.168.60.1.

7.  Verified all 6 cameras obtained DHCP leases in the 192.168.60.0/24 range with correct gateway.

8.  Built and applied extended ACL CAMERA-ISOLATION on the Vlan60 SVI - permits traffic to Server01 (192.168.40.10) and DHCP (UDP 67/68), denies traffic to Clinical, Admin, Billing, and Guest subnets, permits all other traffic (WAN).

9.  Verified isolation via live traffic test - ping from a camera to Server01 succeeded, ping from a camera to a Clinical workstation failed as expected. Note: Packet Tracer's show ip interface command displayed Inbound access list as not set even though the ACL was actively enforcing - verified via actual ping tests rather than relying on the show command output.

Implementation Steps - Item 10 (Guest Network Isolation)

1.  Built extended ACL GUEST-ISOLATION on SW-CORE denying traffic from 192.168.99.0/24 to Clinical (192.168.10.0/24), Admin (192.168.20.0/24), Billing (192.168.30.0/24), IT (192.168.40.0/24), and Cameras (192.168.60.0/24).

2.  Permitted UDP ports 67 and 68 ahead of the final rule so guest devices retain DHCP renewal capability.

3.  Applied a final permit ip any any to allow WAN-bound traffic (203.0.113.1) for all other guest activity.

4.  Applied the ACL inbound on the Vlan 99 SVI.

5.  Verified from the guest wireless laptop - ping to Server01 (192.168.40.10) failed as expected, ping to simulated WAN (203.0.113.1) succeeded.

Command Reference

VLAN 60 and SW-CORE trunk configuration:

vlan 60 name Security_Cameras exit interface vlan 60 ip address 192.168.60.1 255.255.255.0 ip helper-address 192.168.40.10 no shutdown exit interface gigabitEthernet1/0/8 description SW-SECURITY-TRUNK switchport trunk allowed vlan 10,20,30,40,60,99 switchport mode trunk

Camera isolation ACL:

ip access-list extended CAMERA-ISOLATION permit ip 192.168.60.0 0.0.0.255 host 192.168.40.10 permit udp 192.168.60.0 0.0.0.255 any eq 67 permit udp 192.168.60.0 0.0.0.255 any eq 68 deny ip 192.168.60.0 0.0.0.255 192.168.10.0 0.0.0.255 deny ip 192.168.60.0 0.0.0.255 192.168.20.0 0.0.0.255 deny ip 192.168.60.0 0.0.0.255 192.168.30.0 0.0.0.255 deny ip 192.168.60.0 0.0.0.255 192.168.99.0 0.0.0.255 permit ip any any exit interface vlan 60 ip access-group CAMERA-ISOLATION in

Guest isolation ACL:

ip access-list extended GUEST-ISOLATION deny ip 192.168.99.0 0.0.0.255 192.168.10.0 0.0.0.255 deny ip 192.168.99.0 0.0.0.255 192.168.20.0 0.0.0.255 deny ip 192.168.99.0 0.0.0.255 192.168.30.0 0.0.0.255 deny ip 192.168.99.0 0.0.0.255 192.168.40.0 0.0.0.255 deny ip 192.168.99.0 0.0.0.255 192.168.60.0 0.0.0.255 permit udp 192.168.99.0 0.0.0.255 any eq 67 permit udp 192.168.99.0 0.0.0.255 any eq 68 permit ip any any exit interface vlan 99 ip access-group GUEST-ISOLATION in

Verification

| Verification Check | Result | Notes |
| --- | --- | --- |
| VLAN 60 created and SVI up | PASS | 192.168.60.1/24 confirmed up/up |
| SW-SECURITY trunk to SW-CORE | PASS | Confirmed via show interfaces trunk on both switches |
| All 6 cameras connected in VLAN 60 | PASS | Confirmed via show interfaces status |
| Cameras obtain DHCP lease | PASS | 192.168.60.x range confirmed after DHCP scope added |
| Camera to Server01 reachable | PASS | Ping successful - explicit permit working |
| Camera to Clinical blocked | PASS | Ping failed as expected - explicit deny working |
| Guest to Server01 blocked | PASS | Ping failed as expected from guest laptop |
| Guest to WAN still reachable | PASS | Ping to 203.0.113.1 successful |
| RFID reader and server reachable on VLAN 60 | PASS | Confirmed via test laptop ping to .1, .11, .17 - 0% loss |
| RFID reader registers with server | PASS | Resolved after creating admin account via server web interface |
| Authorized card grants door access | PASS | Verified with wconaway and wturner cards |

Item 11 - RFID Server Room Access Control

| Field | Detail |
| --- | --- |
| Devices Added | RFID card reader (RR-Server_RM) and a dedicated registration server (Server0) - both connected to SW-SECURITY in VLAN 60 |
| Purpose | Physical access control for the server room door - badge-based entry restricted to authorized staff |
| Authorized Cards | RFID cards issued to Willie Conaway (wconaway) and Will Turner (wturner) |
| Reader IP | 192.168.60.11 (DHCP, VLAN 60) |
| Registration Server IP | 192.168.60.17 (DHCP, VLAN 60) - IoT Registration Service enabled |

Troubleshooting - RFID Reader Not Responding to Cards

After adding the RFID reader and registration server, card scans produced no response - the door did not unlock and no activity appeared on the server. The following diagnostic steps were used to isolate the cause:

1.  Confirmed both devices held valid DHCP leases in the 192.168.60.0/24 range - reader at 192.168.60.11, server at 192.168.60.17.

2.  Added a temporary laptop to the VLAN 60 segment and ran ping tests to the gateway (192.168.60.1), the registration server (192.168.60.17), and the reader (192.168.60.11) - all four tests returned 0% packet loss, ruling out any Layer 1, 2, or 3 network fault.

3.  Confirmed the registration server's IoT service was toggled On under Services - IoT.

4.  Confirmed the reader's IoT Server configuration was set to Remote Server, with the Server Address field pointing at the registration server's correct IP (192.168.60.17), and Username/Password fields set to admin/admin.

5.  Clicked Connect on the reader's configuration panel multiple times with no visible error and no entry appearing in the server's registered device list.

6.  ROOT CAUSE IDENTIFIED: the registration server's IoT service being toggled On does not create a usable account by itself. The server requires a one-time admin account to be created through its own web interface (HTTP) before any IoT device can successfully authenticate against it. Without that account, the Connect handshake from the reader fails silently with no error message in Packet Tracer's simulated environment.

7.  Resolution: opened a web browser session to the registration server's IP (192.168.60.17), used the Sign Up option to create the admin/admin account directly on the server, then returned to the reader's configuration and clicked Connect again.

8.  Verified the reader successfully registered with the server and responded correctly to an authorized card scan.

KEY INSIGHT: An IoT service showing as On does not guarantee it is usable. Packet Tracer's registration server requires a manually created account via its web interface before any device can authenticate, even with the service toggle enabled and correct admin/admin credentials entered on the client side. This is a good general lesson for real-world IoT and access control deployments as well - a running service is not the same as a configured, ready-to-authenticate service.

Recommendations

1.  Document this configuration in the master network diagram and update the portfolio tracker to reflect the expanded topology (SW-SECURITY, VLAN 60, both new ACLs).

2.  Apply the same isolation logic to any future IoT devices added to the network (badge readers, HVAC controllers, etc.) - any non-traditional endpoint should default to a restricted VLAN unless explicitly required otherwise.

3.  Periodically audit ACLs on SW-CORE as new VLANs or subnets are added, to confirm isolation rules are not silently bypassed by a missing deny line.

4.  Note for future Packet Tracer work: show ip interface vlan X may not reliably display Inbound access list status in this simulator version - always confirm ACL enforcement via live ping tests rather than trusting the show command alone.

5.  For any future IoT device added to the network (badge readers, sensors, etc.), create the registration server's web-based admin account immediately after enabling the IoT service, before attempting to connect any client device - this avoids the silent connection failure documented above.

6.  Consider adding a badge access log review to the security audit cycle (SEC-005) once the camera and RFID systems are both fully operational, to confirm physical access events are being captured alongside network events.

Technician Sign-Off

| Analyst | Willie Conaway (wconaway) | Time to Resolve | Single session |
| --- | --- | --- | --- |
| Ticket | PHC-NET-006 | Hardening Plan | All items now complete |
| Ticket Closed | 06/20/2026 | Next Action | Continue Monitoring |

This ticket is part of the Pinnacle Health Clinic IT Portfolio - a simulated healthcare IT environment built for professional development and portfolio documentation.

Pinnacle Health Clinic - NOC Network Incident Documentation - Confidential